OSPF (Open Shortest Path First)

What is OSPF?

Open Shortest Path First (OSPF) is one of the Interior Gateway Protocol (IGP) which helps to find the best routing path between the source and the destination router using its own shortest path first (SPF) algorithm. It is a Link state routing protocol which is used to distribute routing information about data packets within a large Autonomous System.An autonomous system can be divided into areas, these help in reducing the link state advertisements and other overhead traffic that will be otherwise sent to the network.

How it works?

When it is configured, it listens to its neighbors in the networks and it gathers all the link state data available. This data is then used to make a topology map that contains all available paths in the network. This database is saved for use and we call it Link State Database.

Advantages

- The concept of area division is proposed. After the autonomous system is divided into different regions, the summary of routing information between the regions is adopted, which greatly reduces the quantity of routing information to be transmitted. It also makes routing information not expand rapidly as the network scale increases.

- Open Shortest Path First Protocol has full support for subnets.

- OSPF Supports route tagging: In Open Short Path First, routes can be tagged to ease interoperation with random values.

- Good security. OSPF supports interface-based plaintext and md5 authentication.

- OSPF adapts to various scales of networks, up to thousands of units.

Disadvantages

- OSPF allows routers to draw a network map within themselves. Being able to remember and navigate a map requires a heavy processor load, and this protocol holds multiple copies of routing information, which consumes more memory. Because of this it can be expensive.

- IIt is more complex to understand and learn than other Internet Protocols.

RIP VS OSPF

| Attribute | RIP | OSPF |
| --- | --- | --- |
| Convergence | Slow | Fast |
| Network Size | For smallto mediumnetworks | For largeand smallnetworks |
| Need of DeviceResources | Much lessmemoryand CPUintensivethan OSPF | Memoryand CPUintensive |
| Need of NetworkResources | Bandwidthconsuming;wholeroutingtable is sent | Less thanRIP; onlysmallupdates aresent |
| Metric | Based on hop count | Based on bandwidth |
| Design | Flat network | Hierarchical network possible |

Uğurcan Elagöz - 2018556506