1- what is Routing Information Protocol (RIP):

- Standard protocol for the exchange of routing information among gateways and hosts.

- Developed in June 1988 by Charles Hedrick which makes it one of the oldest routing protocols.

2- how it works :

- It's vector routing based protocols.

- It depends on hop counting.

3- versions:

- RIPv1

- RIPv2

- RIPng (next generation)

4- Limitation:

- Hop count can't exceed 15

- Variable subnet masks are not supported by RIPv1.

- without using RMTI, RIP has slow convergence and has many problems.

5- explanations :

- A distance-vector routing protocol in data networks determines the best route for data packets based on the distance which is measured as the number of routes that's a packet has to pass.

- hop counting in RIP :

RIP prevents routing loops by implementing a limit of 15 hops allowed in a path from source to destination.

- on the other hand, this limit also limits the size of networks that RIP can support.

- the hop count of 16 is considered as infinite distance.

- Each RIP router sends its network table to other routers and updates its own table every 30 seconds.

6- RIPv1 is classful so variable-length subnet masks cannot be used with it.

it does not support authentication of update massages so it exposes the network to attacks.

-and RIPv1 is an older version and no longer a much-used routing protocol.

8- RIPv2 is a classless version of RIP meaning that variable-length subnet masking can be

applied freely using it.

- RIPv2 supports authentication of RIPv2 update massages which makes it safer and more reliable than RIPv1.

on the contrary to v1 which depends on broadcast, RIPv2 supports multicast routing updates to reduce resource consumption.

9- RIPng (next generation) works basically in the same manner as RIPv2 but it works only on IPv6 networks.